# Revision & Date : PVFL30103

# PVFL Series

## Pen Type Visual Fault Locator

Special Features :

Visual Fault Locator is used for locating broken point in fibers

Visual Fault Locator is equipped with a 650nm visible laser diode

Can be operated in continuous or pulse modes

Available 2.5mm and 1.25mm ferrule both

Power switch on the end for battery on / off

Push button operate CW / Pulse / Pause

Light Weight

Specifications :

| Central Wavelength | 650 +/- 5 nm |
| --- | --- |
| Fiber Compatibility | Multimode and Singlemode |
| Output Power | >0.5 mW(-3dBm) into Single-mode fiber |
| Output Port | Universal accept any connector with 2.5mm ferrule |
| Output Mode | Continuous or Flashing (3Hz) |
| Spectral Width | < 20 nm |
| Transmission | CW or 2.5 Hz pulse |
| Batteries | 2 x 1.5V AAA Alkaline; 40 hours typical in continuous mode |
| Operating Temp. | 0℃ to 40 ℃ 0 to 95 ℃ RH (non-condensing) |
| Storage Temp. | -20℃ to +60℃ 0 to 95 ℃ RH (non-condensing) |
| Detection Distance | < 3 Km |
| Dimension / Weight | 13.5(dia) mm X 146(length) mm / 80 g (include battery) |
| Light Source | Class 3A laser diode |

Ordering Information :

P/N: PVFL : For 2.5mm Ferrule (standard),  TIP-1.25: For 1.25mm Ferrule (option)

TheFibers

C R E AT I V E  F I B E R S

TheFibers Inc.

A-1109 Keumkang Penterium IT Tower, 282 Hagui-ro, Anyang-City, Korea

Tel: +82-31-381-6108  Fax: +82-31-381-6109  sales@thefibers.com